At the end of the day, disinfect the dismantled nebuliser handset (including the aerosol head) immediately after cleaning for about 15 minutes

Disinfection methods:

- Boil

- Steam steriliser

Drying

Place all components on a dry absorbent surface and air dry for 4 hours

Storing

Keep in a dust free place protected from contamination

Replacement parts

Aerosol heads are recommended to be replaced every

3 - 6 months and handsets every 12 months on average

- Take the nebuliser apart completely. Wash immediately with warm tap water and washing up liquid for 5 minutes

- After washing the aerosol head, rinse the front and back of the aerosol head for approx. 1 minute under warm running tap water.

- Clean the control unit and cord with a damp cloth

E-Flow® Cleaning Instructions

E-Flow® Disinfection